-- Component Declaration for the Unit Under Test (UUT)

COMPONENT uadder PORT(

A : IN std_logic_vector(3 downto 0); B : IN std_logic_vector(3 downto 0); C : OUT std_logic_vector(3 downto 0)

);

END COMPONENT;

--Inputs

signal A : std_logic_vector(3 downto 0) := (others => '0'); signal B : std_logic_vector(3 downto 0) := (others => '0');

--Outputs

signal C : std_logic_vector(3 downto 0);

-- No clocks detected in port list. Replace <clock> below with

-- appropriate port name

--constant <clock>_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT) uut: uadder PORT MAP (

A => A,

B => B, C => C

);

-- Clock process definitions

--<clock>_process :process

--begin

--<clock> <= '0';

--wait for <clock>_period/2;

--<clock> <= '1';

--wait for <clock>_period/2;

--end process;

-- Stimulus process stim_proc: process begin

-- hold reset state for 100 ns. A<="0111";B<="0010"; wait for 100 ns;

--wait for <clock>_period*10;

-- insert stimulus here

wait;

end process;

END;

-- Code your design here library IEEE;

use IEEE.std_logic_1164.all;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx primitives in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity uadder is

Port ( A : in STD_LOGIC_VECTOR (3 downto 0); B : in STD_LOGIC_VECTOR (3 downto 0); C : out STD_LOGIC_VECTOR (3 downto 0));

end uadder;

architecture Dataflow of uadder is

begin

C <= A - B;

end Dataflow;

Output :

Discussion :

Assignment 4: Implementation of binary to Gray converter and Gray to Binary converter using XILINX ISE

Aim: Software used:

| Property Name | XILINX ISE |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language | VHDL |

Gray to Binary:

Theory :

The reflected binary code or Gray code is an ordering of the binary numeral system such that two successive values differ in only one bit (binary digit). This is very simple method to get Binary number from Gray code. These are following steps for n-bit binary numbers −

- The Most Significant Bit (MSB) of the binary code is always equal to the MSB of the given binary number.

- Other bits of the output binary code can be obtained by checking gray code bit at that index. If current gray code bit is 0, then copy previous binary code bit, else copy invert of previous binary code bit.

Truth table: (Gray to Binary)

| Gray |  |  |  | Binary |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G3 | G2 | G1 | G0 | B3 | B2 | B1 | B0 |
| 0 | 0 | 0 | 0 |  |  |  |  |
| 0 | 0 | 0 | 1 |  |  |  |  |
| 0 | 0 | 1 | 0 |  |  |  |  |
| 0 | 0 | 1 | 1 |  |  |  |  |
| 0 | 1 | 0 | 0 |  |  |  |  |
| 0 | 1 | 0 | 1 |  |  |  |  |
| 0 | 1 | 1 | 0 |  |  |  |  |
| 0 | 1 | 1 | 1 |  |  |  |  |
| 1 | 0 | 0 | 0 |  |  |  |  |
| 1 | 0 | 0 | 1 |  |  |  |  |
| 1 | 1 | 1 | 0 |  |  |  |  |
| 1 | 0 | 1 | 1 |  |  |  |  |
| 1 | 0 | 1 | 0 |  |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |  |
| 1 | 1 | 0 | 1 |  |  |  |  |
| 1 | 1 | 1 | 1 |  |  |  |  |

Dataflow Model CODE:

Design

library IEEE;
use IEEE.STD_LOGIC_1164.ALL;


entity gtb is
    Port ( g : in  STD_LOGIC_VECTOR (3 downto 0);
           b : inout  STD_LOGIC_VECTOR (3 downto 0));
end gtb;

architecture Dataflow of gtb is

begin
b(3)<=g(3);
b(2)<=b(3) xor g(2);
b(1)<=b(2) xor g(1);
b(0)<=b(1) xor g(0);

end Dataflow;

TestBench

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

 
ENTITY tb_gtb IS
END tb_gtb;
 
ARCHITECTURE behavior OF tb_gtb IS
 
    COMPONENT gtb
    PORT(
         g : IN  std_logic_vector(3 downto 0);
         b : INOUT  std_logic_vector(3 downto 0)
        );
    END COMPONENT;

   signal g : std_logic_vector(3 downto 0) := (others => '0');

   signal b : std_logic_vector(3 downto 0);
BEGIN
 
   uut: gtb PORT MAP (
          g => g,
          b => b
        );

   stim_proc: process
   begin       
      wait for 100 ns;   

     g<="1100";


      wait;
   end process;

END;

Code:

Behavioral Model code:

Design
entity gtb_bev is
    Port ( g : in  STD_LOGIC_VECTOR (3 downto 0);
           b : inout  STD_LOGIC_VECTOR (3 downto 0));
end gtb_bev;

architecture Behavioral of gtb_bev is

begin
process(g,b)
begin
b(3)<=g(3);
for i in 2 downto 0 loop
if(b(i+1)=g(i)) then
b(i)<='0';
else
b(i)<='1';
end if;
end loop;
 end process;

end Behavioral;

Testbench

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_gtb_bev IS
END tb_gtb_bev;
 
ARCHITECTURE behavior OF tb_gtb_bev IS
 
    COMPONENT gtb_bev
    PORT(
         g : IN  std_logic_vector(3 downto 0);
         b : INOUT  std_logic_vector(3 downto 0)
        );
    END COMPONENT;
    

   signal g : std_logic_vector(3 downto 0) := (others => '0');

   signal b : std_logic_vector(3 downto 0);

BEGIN

   uut: gtb_bev PORT MAP (
          g => g,
          b => b
        );
          -- Stimulus process
   stim_proc: process
   begin       
      wait for 100 ns;   
    g<="1010";

      wait;
   end process;

END;

Output :

Binary to Gray:

Theory :

This is very simple method to get Gray code from Binary number. These are following steps for n-bit binary numbers −

- The most significant bit (MSB) of the Gray code is always equal to the MSB of the given Binary code.

- Other bits of the output Gray code can be obtained by XORing binary code bit at the index and previous index.

Truth table: (Binary to Gray)

| Binary |  |  |  | Gray |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3 | B2 | B1 | B0 | G3 | G2 | G1 | G0 |  |

Behavioral Model:

Here, the gate is designed, using the truth table. It simulates the behavior of the output of the gate, upon taking different Boolean variables as inputs. The Behavioral Architecture is used in this case.

Data flow Model:

Here, the gate is constructed, using the Dataflow Architecture. The gate is designed, using it’s Boolean function

Code:

Behavioral Model code:

Design
library IEEE;
use IEEE.STD_LOGIC_1164.ALL;


entity btg is
    Port ( b : in  STD_LOGIC_VECTOR (3 downto 0);
           g : out  STD_LOGIC_VECTOR (3 downto 0));
end btg;

architecture Behavioural of btg is

begin
process(b)
begin
g(3)<=b(3);
for i in 0  to 2 loop
if(b(i+1)=b(i)) then
g(i)<='0';
else
g(i)<='1';
end if;
end loop;
end process;

end Behavioural;

Testbench:
LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_btg IS
END tb_btg;
 
ARCHITECTURE behavior OF tb_btg IS

    COMPONENT btg
    PORT(
         b : IN  std_logic_vector(3 downto 0);
         g : OUT  std_logic_vector(3 downto 0)
        );
    END COMPONENT;

   signal b : std_logic_vector(3 downto 0) := (others => '0');

   signal g : std_logic_vector(3 downto 0);

BEGIN

   uut: btg PORT MAP (
          b => b,
          g => g
        );

   stim_proc: process
   begin       

      wait for 100 ns;   

      b<="1100";

      wait;
   end process;

END;

Dataflow Model Code:

Design

library IEEE;
use IEEE.STD_LOGIC_1164.ALL;


entity btg is
    Port ( b : in  STD_LOGIC_VECTOR (3 downto 0);
           g : out  STD_LOGIC_VECTOR (3 downto 0));
end btg;

architecture Dataflow of btg is

begin

g(3)<=b(3);
g(2)<=b(3) xor b(2);
g(1)<=b(2) xor b(1);
g(0)<=b(1) xor b(0);


end Dataflow;



Testbench
LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_btg IS
END tb_btg;
 
ARCHITECTURE behavior OF tb_btg IS

    COMPONENT btg
    PORT(
         b : IN  std_logic_vector(3 downto 0);
         g : OUT  std_logic_vector(3 downto 0)
        );
    END COMPONENT;

   signal b : std_logic_vector(3 downto 0) := (others => '0');

   signal g : std_logic_vector(3 downto 0);

BEGIN

   uut: btg PORT MAP (
          b => b,
          g => g
        );

   stim_proc: process
   begin       

      wait for 100 ns;   

      b<="1000";

      wait;
   end process;

END;

Output :

Discussion:

Gray codes are very useful in the normal sequence of binary numbers generated by the hardware that may cause an error or ambiguity during the transition from one number to the next. So, the Gray code can eliminate this problem easily since only one bit changes its value during any transition between two numbers.

Assignment 5: Implementation of 4bit comparator, 2:4 Decoder,3:8 Decoder, 8bit Multiplier using XILINX ISE

Aim: Software used:

| Property Name | XILINX ISE |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language | VHDL |

4 bit Comparator: Theory:

Code:

Behavioral Model Code:

Output:

2:4 Decoder: Truth table:

| I1 | I0 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 |

Dataflow Model:

Here, the gate is constructed, using the Dataflow Architecture. The gate is designed, using it’s Boolean function

Code:

DATAFLOW:

Design
library IEEE;
use IEEE.STD_LOGIC_1164.ALL;

entity decoder is
    Port ( i0 : in  STD_LOGIC;
           i1 : in  STD_LOGIC;
           d0 : out  STD_LOGIC;
           d1 : out  STD_LOGIC;
           d2 : out  STD_LOGIC;
           d3 : out  STD_LOGIC);
end decoder;

architecture Dataflow of decoder is

begin
d0<=(not i0)and(not i1);
d1<=( i0)and(not i1);
d2<=(not i0)and(i1);
d3<=(i0)and( i1);

end Dataflow;

Testbench

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

 
ENTITY tb_decoder IS
END tb_decoder;
 
ARCHITECTURE behavior OF tb_decoder IS

    COMPONENT decoder
    PORT(
         i0 : IN  std_logic;
         i1 : IN  std_logic;
         d0 : OUT  std_logic;
         d1 : OUT  std_logic;
         d2 : OUT  std_logic;
         d3 : OUT  std_logic
        );
    END COMPONENT;

   signal i0 : std_logic := '0';
   signal i1 : std_logic := '0';

   signal d0 : std_logic;
   signal d1 : std_logic;
   signal d2 : std_logic;
   signal d3 : std_logic;

 
BEGIN
 
   uut: decoder PORT MAP (
          i0 => i0,
          i1 => i1,
          d0 => d0,
          d1 => d1,
          d2 => d2,
          d3 => d3
        );

   stim_proc: process
   begin       

      wait for 100 ns;   
        i0<='0';wait for 100ns;
        i1<='1';
  
      wait;
   end process;

END;

Output :

3:8 Decoder: Truth table:

| I2 | I1 | I0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Code:

Design

library IEEE;
use IEEE.STD_LOGIC_1164.ALL;
USE ieee.std_logic_arith.all;
USE ieee.std_logic_unsigned.all;


entity decoder_behav is
    Port ( i : in  STD_LOGIC_VECTOR(2 downto 0);
           d : out  STD_LOGIC_VECTOR (7 downto 0));
end decoder_behav;

architecture Behavioral of decoder_behav is
begin
process(i)
variable s:integer;
begin

s:=CONV_INTEGER(i);
for i in 7 downto 0 loop
if(i=s) then
d(i)<='1';
else
d(i)<='0';
end if;
end loop;
end process;

end Behavioral;



Testbench

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

 
ENTITY tb_decoder IS
END tb_decoder;
 
ARCHITECTURE behavior OF tb_decoder IS

 
    COMPONENT decoder_behav
    PORT(
         i : IN  std_logic_vector(2 downto 0);
         d : OUT  std_logic_vector(7 downto 0)
        );
    END COMPONENT;

   signal i : std_logic_vector(2 downto 0) := (others => '0');

   signal d : std_logic_vector(7 downto 0);

BEGIN

   uut: decoder_behav PORT MAP (
          i => i,
          d => d
        );
   stim_proc: process
   begin       

      wait for 100 ns;   

  i<="011";wait for 100 ns;

      wait;
   end process;

END;

Output :

8 Bit Multiplier: Code:

Output:

Discussion:

Assignment 6: Implementation of 2:4 Decoder (using case), 4:2 Encoder (Dataflow & using case), 8:3 Encoder (using loop)

Aim: Software used:

| Property Name |  |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language |  |

2:4 Decoder: Theory:

Truth table:

| I1 | I0 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- |

Behavioral Model: (using case)

Output :

4:2 Encoder: Theory :

Truth Table:

| I3 | I2 | I1 | I0 | E1 | E0 |
| --- | --- | --- | --- | --- | --- |

Dataflow Model:

Dataflow Code:

Output :

Behavioral Code:

Output :

8:3 Encoder: Truth Table:

| I7 | I6 | I5 | I4 | I3 | I2 | I1 | I0 | E2 | E1 | E0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Behavioral Code:

Output :

Discussion :

Assignment 7: Implementation of 2:1 MUX(Dataflow), 4:1 MUX (Dataflow, if-else and using Case) and 1:4 Demux (Using Dataflow and case) using XILINX ISE.

Software used:

| Property Name |  |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language |  |

Theory :

2:1 MUX:

Truth table:

| INPUT |  |  | OUTPUT |
| --- | --- | --- | --- |
| S | I1 | I0 | M |

Dataflow Model:

Code:

Dataflow Model:

4:1 MUX:

Truth Table:

| INPUT |  |  |  |  |  | OUTPUT |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | S0 | I3 | I2 | I1 | I0 | M |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 |

Dataflow Mode

Dataflow Model Code:

library IEEE;
use IEEE.STD_LOGIC_1164.ALL;


entity mux4x1 is
    Port ( s1 : in  STD_LOGIC;
           s0 : in  STD_LOGIC;
           i3 : in  STD_LOGIC;
           i2 : in  STD_LOGIC;
           i1 : in  STD_LOGIC;
           i0 : in  STD_LOGIC;
           m : out  STD_LOGIC);
end mux4x1;

architecture Dataflow of mux4x1 is

begin
m<=((not s0) and (not s1) and i0) or ((not s1) and s0 and i1)
 or ( s1 and (not s0) and i2) or ( s0 and s1 and i3);
end Dataflow;

TESTBENCH

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_mux4x1 IS
END tb_mux4x1;
 
ARCHITECTURE behavior OF tb_mux4x1 IS
 
    -- Component Declaration for the Unit Under Test (UUT)
 
    COMPONENT mux4x1
    PORT(
         s1 : IN  std_logic;
         s0 : IN  std_logic;
         i3 : IN  std_logic;
         i2 : IN  std_logic;
         i1 : IN  std_logic;
         i0 : IN  std_logic;
         m : OUT  std_logic
        );
    END COMPONENT;

   signal s1 : std_logic := '0';
   signal s0 : std_logic := '0';
   signal i3 : std_logic := '0';
   signal i2 : std_logic := '0';
   signal i1 : std_logic := '0';
   signal i0 : std_logic := '0';

   signal m : std_logic;

BEGIN

   uut: mux4x1 PORT MAP (
          s1 => s1,
          s0 => s0,
          i3 => i3,
          i2 => i2,
          i1 => i1,
          i0 => i0,
          m => m
        );

   stim_proc: process
   begin       

      wait for 100 ns;   
s1<='1';s0<='0';i3<='0';i2<='1';i1<='0';i0<='0';

      wait;
   end process;

END;

Output :

Behavioral Model code: (Using if-else)

entity mux4x1_be is
    Port ( s : in  STD_LOGIC_VECTOR (1 downto 0);
           i : in  STD_LOGIC_VECTOR (3 downto 0);
           m : out  STD_LOGIC);
end mux4x1_be;

architecture Behavioral of mux4x1_be is

begin
process(i,s)
begin
if(s(0)=s(1)) then
if(s(0)='0') then
m<=i(0);
else
m<=i(3);
end if;
else
if(s(0)='1') then
m<=i(1);
else
m<=i(2);
end if;
end if;
end process;

end Behavioral;

testbench

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_mux4x1 IS
END tb_mux4x1;
 
ARCHITECTURE behavior OF tb_mux4x1 IS

    COMPONENT mux4x1_be
    PORT(
         s : IN  std_logic_vector(1 downto 0);
         i : IN  std_logic_vector(3 downto 0);
         m : OUT  std_logic
        );
    END COMPONENT;

   signal s : std_logic_vector(1 downto 0) := (others => '0');
   signal i : std_logic_vector(3 downto 0) := (others => '0');

   signal m : std_logic;

BEGIN

   uut: mux4x1_be PORT MAP (
          s => s,
          i => i,
          m => m
        );

   stim_proc: process
   begin       

      wait for 100 ns;   

      s<="01";i<="0010";

      wait;
   end process;

END;

Output:

1:4 De mux: Theory :

Truth Table:

| Data Input | Select Input |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| D | S1 | S0 | Y3 | Y2 | Y1 | Y0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |

Dataflow Model:

Dataflow Code:

library IEEE;
use IEEE.STD_LOGIC_1164.ALL;



entity dmux1x4 is
    Port ( d : in  STD_LOGIC;
           s1 : in  STD_LOGIC;
           s0 : in  STD_LOGIC;
           o : out  STD_LOGIC_VECTOR (3 downto 0));
end dmux1x4;

architecture Dataflow of dmux1x4 is

begin
o(0)<=d and (not s0) and (not s1);
o(1)<=d and (not s1) and  s0;
o(2)<=d and  s1 and (not s0);
o(3)<=d and  s0 and  s1;


end Dataflow;


LIBRARY ieee;
USE ieee.std_logic_1164.ALL;
 

 
ENTITY tb_demux IS
END tb_demux;
 
ARCHITECTURE behavior OF tb_demux IS
 
    -- Component Declaration for the Unit Under Test (UUT)
 
    COMPONENT dmux1x4
    PORT(
         d : IN  std_logic;
         s1 : IN  std_logic;
         s0 : IN  std_logic;
         o : OUT  std_logic_vector(3 downto 0)
        );
    END COMPONENT;

   signal d : std_logic := '0';
   signal s1 : std_logic := '0';
   signal s0 : std_logic := '0';

   signal o : std_logic_vector(3 downto 0);

BEGIN

   uut: dmux1x4 PORT MAP (
          d => d,
          s1 => s1,
          s0 => s0,
          o => o
        );

   stim_proc: process
   begin       

      wait for 100 ns;   

     d<='1'; s1<='1';s0<='0';



      wait;
   end process;

END;

Output:

Discussion:

In this assignment we learned about multiplexer and de multiplexer and learned about  the implementation in both dataflow and behavioral.

Assignment 8: Implementation of Full Adder using Half Adder (using Structural Method), 4 bit Parallel Adder (using Structural Method).

Aim : Software used:

| Property Name |  |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |

| Top-level source type |  |
| --- | --- |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language |  |

Full Adder: Theory:

Truth Table:

| INPUT |  |  | OUTPUT |  |
| --- | --- | --- | --- | --- |
| A | B | Cin | S | Cout |

Structural Model

Code:

Output:

4 Bit Parallel Adder: Theory:

Structural Model:

Code:

Output:

Discussion:

Assignment 9: Implementation of 2:1 MUX (using Structural Method) & 4:1 MUX using 2:1 MUX (using Structural Method) in XILINX ISE

Aim :

Software used:

| Property Name |  |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language |  |

Theory :

2:1 MUX

Truth table:

| INPUT |  |  | OUTPUT |
| --- | --- | --- | --- |
| S | I1 | I0 | Y |

Structural Model:

Code : (Structural method)

Output :

4:1 MUX

Truth table:

| S1 | S0 | Y |
| --- | --- | --- |

Structural Model:

Code : (Structural)

Output :

Discussion :

Assignment 10: Implementation of SR flip flop, JK flip flop, D flip flop, T flip flop (using if

–else) in XILINX ISE Aim :

Software used:

| Property Name |  |
| --- | --- |
| Device family |  |
| Device |  |
| Package |  |
| Speed |  |
| Top-level source type |  |
| Synthesis Tool |  |
| Simulator |  |
| Preferred Language |  |

Theory :

SR flip flop: Truth table:

| CLK | S | R | Q | Q’ |
| --- | --- | --- | --- | --- |

Dataflow model:

Code :

Output :

JK flip flop:

Truth table:

| CLK | J | K | Q | Q’ |
| --- | --- | --- | --- | --- |

Dataflow model:

Code :

Output :

D flip flop: Truth table:

| CLK | D | Q | Q’ |
| --- | --- | --- | --- |

Dataflow model:

Code :

Output :

T flip flop: Truth table:

| CLK | T | Q | Q’ |
| --- | --- | --- | --- |

Dataflow model:

Code :

Output :

Discussion: